LAB4;

19K-1084

BSE-5A

EXERCISE;

Q1;

Socket programming is a way of connecting two nodes on a network to communicate with

each other. One socket(node) listens on a particular port at an IP, while other socket

reaches out to the other to form a connection. Server forms the listener socket while client

reaches out to the server.

Q2;

LISTEN[ PARAMETERS]

PARAMERTERS DECIDES HOW MANY CONNECTIONS SHOULD BE MADE.

TO MAKE A LISTEN ONLT CONNECTION ;… LISTEN[1] WILL BE PASSED.

Q3;

TCPSERVER.PY

from socket import *

serverPort = 12000

serverSocket = socket(AF_INET,SOCK_STREAM)

serverSocket.bind(('',serverPort))

serverSocket.listen(3)

print('The server is ready to receive')

while True:

connectionSocket, addr = serverSocket.accept()

sentence = connectionSocket.recv(1024).decode()

capitalizedSentence = sentence.upper()

connectionSocket.send(capitalizedSentence.encode())

connectionSocket.close()

TCPCLIENT.py

from socket import *

serverName = 'HAIER-PC'

serverPort = 12000

clientSocket = socket(AF_INET, SOCK_STREAM)

clientSocket.connect((serverName,serverPort))

sentence = input('Input lowercase sentence:')

clientSocket.send(sentence.encode())

modifiedSentence = clientSocket.recv(1024)

print('From Server: ', modifiedSentence.decode())

clientSocket.close()

Q4;

Figure1;

Figure 2;

Pdu-information;

WIRESHARK ANSWER;

- UDP USER DATAGRAM PROTOCOL

- 0.0.0.0 REPRESENTS LOCAL HOST/SERVER POOL.

- 255.255.255.255 CLASS E EXPERIMENTAL CLASS.

- 0X56F415ED

- 5 PIECES ARE;

- MESSAGE TYPE; BOOT REQUEST[1] SINGLE CONNECTION

- HARDWARE-TYPE; ETERNET [0X01]

- HARDWARE LENGTH 6

- HOPS

- TRANSACTION ID; 0X56F415ED

- SRC PORT; 68, DEST PORT; 67

- UDP.